# Arch-Hex Grid Sole

Draft invention disclosure and claim structure — Patent 1 (Boot)

DRAFT — FOR PATENT ATTORNEY REVIEW ONLY. Not filing-ready. Dutch filing at mijn.rvo.nl requires claims in Dutch and formal claim-drafting review — this document is the technical invention disclosure and a starting claim structure for your attorney to refine, not a document to submit as-is.

# 1. Field of the invention

A sole structure for a liquid-filled wearable warming device (a hot water boot) that keeps an internal water channel open under standing load, where a conventional flat bladder would compress shut.

# 2. Background

A hot water bottle placed under a seated foot works because the foot's weight does not fully compress the bladder against a hard floor. Worn as a boot and stood on, a flat water bladder is compressed flat by body weight exactly where the wearer needs warmth most, cutting off water flow and defeating the purpose. No identified product in the hot-water-warmer category solves this for a worn, standable device — see the accompanying Prior Art Search Request.

# 3. Summary of the invention

The sole is built as a field of hollow, upright hexagonal columns rising from a base layer, each capped with a solid flat top. The columns are arranged in three density zones — higher density (narrower columns) under the heel and toe, lower density (wider columns) under the arch — and spaced so that an open fluid channel persists beneath the water bladder even at full standing load. A critical structural detail is the column wall thickness (4mm), which is what resists the channel collapsing shut. The same channel-preservation principle is also the conceptual basis for the separate PCM-cartridge invention (see Patent Brief v3.0): keep the heat-transfer medium unobstructed under load.

# 4. Draft claims

1.  A sole structure for a liquid-filled wearable warming device, comprising: a base layer; and a plurality of hollow, upright hexagonal support columns extending from the base layer, each column having a closed top cap and an open interior in fluid communication with a liquid-containing bladder positioned above the sole structure; wherein the support columns are spaced apart to define open fluid channels beneath the bladder that remain open when the sole structure is subjected to a static load corresponding to a standing human foot.

2.  The sole structure of claim 1, wherein the support columns have a wall thickness of substantially 4mm, configured to resist collapse of the fluid channels under the static load.

3.  The sole structure of claim 1, wherein the support columns are arranged in three zones of differing density: a heel zone and a toe zone having columns of a first width, and an arch zone positioned between the heel and toe zones having columns of a second width greater than the first width.

4.  The sole structure of claim 3, wherein the first width is substantially 15mm flat-to-flat and the second width is substantially 20 to 22mm flat-to-flat.

5.  The sole structure of claim 1, wherein the fluid channels have a height of substantially 4 to 5mm and remain open at a static load of at least 150kg.

6.  The sole structure of claim 3, further comprising a flat arch bridge spanning between adjacent support columns in the arch zone, the arch bridge having a rise-to-span ratio of between 0.2 and 0.4.

7.  The sole structure of claim 1, formed from a food-grade polypropylene material.

# 5. Key technical parameters (for attorney reference)

| Parameter | Value | Relevant to |
| --- | --- | --- |
| Column width — heel/toe | 15mm flat-to-flat, +0/−0.2mm | Claim 4 |
| Column width — arch | 20–22mm flat-to-flat | Claim 4 |
| Column height | 8mm ±0.1mm | Claim 1 |
| Column wall thickness | 4mm ±0.1mm — critical load-bearing dimension | Claim 2 |
| Fluid channel height | 4–5mm, open beneath every column | Claim 5 |
| Load rating | 200kg+ static, tested to 150kg standard | Claim 5 |
| Material | Food-grade PP — FDA 21 CFR 177.1520 | Claim 7 |

# 6. Figures required

- Isometric view of the hex-column grid showing all three density zones.

- Cross-section through a single column showing the closed cap, open interior, and fluid channel beneath.

- Cross-section through the arch-zone bridge showing the rise-to-span geometry.

- Load diagram illustrating channel geometry unloaded vs. under a 150kg static load.

# 7. Related filings

This brief covers Patent 1 in the HydroWarm portfolio — an independent invention from the PCM-cartridge system covered in Patent Brief v3.0 (Comfort/Charge). File separately. Send this document together with the Prior Art Search Request memo (HydroWarm_Prior_Art_Search_Request.docx).

Full manufacturing tolerances and sourcing detail for this sole are in the companion engineering document, HydroWarm_ArchHex_Boot_Factory_Spec_v1.docx — that document is for the factory, this one is for the attorney; keep them separate so factory tooling changes don't require re-touching claim language.