HW-COMFORT-SYS

# HydroWarm Comfort

Single-zone PCM slipper system — launching first

| Retail price | Launch window | COGS target | CE standard |
| --- | --- | --- | --- |
| €34–39 | Month 5–7 | €11–15 | IEC 60335-2-81 |

# 1. Overview

Comfort is the first HydroWarm product to reach market. A single-zone phase-change (PCM) gel cartridge sits in a ribbed polypropylene tray and is recharged by an induction plate placed under the desk — no cords to the foot, no resistive heating element, no open flame. It exists to prove the core cartridge-and-induction technology at the lowest possible cost and complexity before Charge and Boot follow it to market.

This specification supersedes the v1.0 factory brief. The single change from v1.0 is the ribbed tray floor (Section 3.2) — the flat floor in v1.0 allowed the cartridge to be crushed under seated foot load and must not be used for tooling.

# 2. Bill of components

| Component | Spec | Material |
| --- | --- | --- |
| Tray | One per foot, houses cartridge + wiring channel | Polypropylene (PP) |
| Tray floor | Ribbed, 3mm ribs at 8mm spacing | PP, moulded integral to tray |
| Cartridge access | 8cm velcro flap, grey minky fabric face | Hook-and-loop + minky textile |
| PCM cartridge | Single zone, heel-to-toe, 22cm | TPU pouch, heat-welded |
| PCM gel fill | 38–40°C phase change | PEG 1000 |
| Induction disc | Bonded to base of cartridge | 430-grade ferromagnetic stainless / carbon steel |
| Induction plate | 45 × 28 × 10mm, 2 coils (1 per foot) | ABS housing, non-ferromagnetic |
| Control | Single 3-position dial, both feet same setting | — |

# 3. PP tray

3.1 Material

Polypropylene, chosen because it is induction-transparent — the coil's field passes through the tray freely to reach the steel disc — and cheap to mould at volume. Aluminium was rejected for this tier: it is reserved for Charge, where its cost is justified by the load-bearing aluminium frame.

3.2 Ribbed floor — critical, v2.0

⚠  Do not tool the flat floor from v1.0. It allows the gel cartridge to be crushed under seated foot load, which is a safety defect, not a cosmetic one.

| Parameter | Value |
| --- | --- |
| Rib height | 3mm |
| Rib spacing | 8mm |
| Cost impact | Near-zero — same tool, revised floor geometry |
| Purpose | Keeps the cartridge unloaded so the gel cannot be compressed flat |

3.3 Future upgrade — not for launch

A Mini Arch-Hex Grid floor variant is documented for a possible Year 2 stand-rated tier of Comfort, extending the same patent family as the Boot sole. It is not part of this build — Comfort ships as a seated-use product only.

# 4. Cartridge and access

4.1 Zones

Single zone, heel to toe, 22cm. One cartridge per foot. This is deliberately the simplest possible configuration — it validates the core PCM + induction technology before Charge introduces independently-regulated heel and toe zones.

4.2 Access mechanism

An 8cm-wide velcro flap in the same grey minky fabric as the rest of the upper. Rated to 5,000+ open/close cycles in soft-goods testing. Magnetic click (used on Charge) and zip closures were both considered and rejected here as unnecessary cost and complexity for an entry product.

# 5. PCM gel fill

| Property | Comfort launch spec |
| --- | --- |
| Material | PEG 1000 (polyethylene glycol) |
| Phase change temperature | 38–40°C |
| Latent heat | ~160 J/g |
| Safety | Food-grade — used in cosmetics and pharma |
| Thermal cycles | 1,000+ stable |
| Appearance | Translucent white / amber when liquid |
| Supplier | Sigma Aldrich or Croda — widely available in the EU |
| COGS per cartridge | €1.20–1.80 |

A fatty-acid eutectic blend (lauric/myristic/palmitic, 36–42°C tunable) is documented as a Year 2 alternative but is not the launch material. Cherry pits are not compatible with this system at all — they are not ferromagnetic and cannot be used with the induction plate.

# 6. Induction plate (Comfort variant)

| Parameter | Spec | Note |
| --- | --- | --- |
| Dimensions | 45 × 28 × 10mm | 2 coils, 1 per foot, single zone each |
| Steel disc material | Ferromagnetic only — carbon steel or 430-grade stainless | NEVER 304 stainless — it is non-magnetic and will not heat at all |
| Plate housing | ABS plastic | Confirmed non-ferromagnetic — no metal reinforcement, which would itself heat and burn |
| Coil frequency | 20–100kHz | Do not guess — confirm exact frequency with the electronics OEM |
| Surface temp (no cartridge) | < 10°C rise above ambient | Returns to within 5°C of ambient within 5 minutes of cartridge removal |
| Detection | Auto on/off — activates only when a ferromagnetic disc is present | Near-zero standby power when no slipper is on the plate |
| Control | Single 3-position dial | Both feet share one setting on Comfort (Charge gets independent per-zone control) |

# 7. Manufacturing and sourcing

Track 1 (China, Guangdong / Zhejiang) is the launch route: COGS €11–15 per system, 90–120 day lead time, for the first 500–1,000 units sold through Bol.com, Amazon and Kickstarter. Track 2 (Portugal soft goods + Poland injection moulding) is a Year 2 move for the EU-premium tier and is not part of the launch plan.

| Component | Sourcing (launch) | Note |
| --- | --- | --- |
| PP tray (ribbed) | China | Injection moulding |
| Cartridge fabric assembly | China | Minky plush, velcro flap |
| PCM gel fill | EU supplier preferred | PEG 1000 from Sigma Aldrich or Croda EU |
| PCM cartridge (TPU pouch) | China | Heat-welded TPU, food-grade |
| Induction steel disc | China | 430-grade ferromagnetic stainless only — not 304 |
| Induction plate PCB + coil | China — electronics OEM | Confirm coil frequency (20–100kHz) with supplier |

# 8. COGS and retail pricing

| Item | Value | Status |
| --- | --- | --- |
| COGS (China track) | €11–15 | Confirmed range — pending first factory quote |
| Retail price | €34–39 | €36 recommended, not locked — confirm after first COGS quote |
| Replacement cartridge (2-pack) | €9 | HW-COMFORT-CART |

# 9. Certification

IEC 60335-2-81 covers appliances for skin/hair heating and applies to the induction plate. Get a quote from DEKRA Netherlands — budget €2,000–4,000 — before committing to first production tooling.

# 10. Do not change without explicit discussion

- The ribbed tray floor (v2.0) — the flat floor was a safety defect, now fixed.

- Steel disc material: ferromagnetic only, 430-grade or carbon steel, never 304 stainless.

- PCM temperature target: 38–40°C, self-regulating via the induction plate.

- Launch sequence: Comfort first, then Charge, then Boot.

- Single-zone cartridge for Comfort — dual-zone is a Charge-only feature.