HW-CHARGE-SGL-SYS  /  HW-CHARGE-DBL-SYS

# HydroWarm Charge

Dual-zone premium system — Single and Double

| Retail price | Launch window | Frame material | CE standard |
| --- | --- | --- | --- |
| €59 (Single) / €69 (Double) | Year 1–2 | Aluminium 6061 alloy | IEC 60335-2-81 |

# 1. Overview

Charge is the premium tier, arriving after Comfort has proven the core PCM-and-induction technology in market. Where Comfort is single-zone and velcro-closed, Charge adds independently-regulated heel and toe zones, a load-bearing aluminium frame that doubles as the heat-conduction path, and tool-free neodymium magnetic cartridge retention. Double is the same system built into one wide slipper for both feet.

Charge shares its underlying PCM chemistry and induction principle with Comfort — this spec covers only what is different: the frame, the dual-zone cartridge, the magnetic retention, the branding window, and the four-coil induction plate.

# 2. Bill of components

| Component | Spec | Material |
| --- | --- | --- |
| Frame | Channel frame, load-bearing + heat conduction | Aluminium 6061 alloy, anodised |
| Cartridge zones | 2 per foot — heel 10×9cm, toe 12×9cm | TPU pouch with magnetic strip |
| Retention | Neodymium magnetic click, 3–5N engage / 5–8N release | N35-grade neodymium, rated to 80°C |
| Branding window | Translucent face, HydroWarm embossed | TPU — amber gel visible through face |
| Induction disc | Bonded to base of EACH cartridge, not the sole | 430-grade ferromagnetic stainless / carbon steel |
| Induction plate | 50 × 30 × 12mm, 4 coils (2 zones × 2 feet) | ABS housing, non-ferromagnetic |
| Indicator | Amber LED while heating, green LED at temperature — independent per zone | — |

# 3. Aluminium frame

6061-alloy channel frame, anodised. It does two jobs at once: it bears the load of a seated or lightly-standing foot, and it conducts heat from the induction-heated disc into the cartridge walls from more than one side — the reason Charge cartridges feel warmer, faster, than Comfort's single-sided tray contact. This is a deliberate upgrade from Comfort's PP tray, which was kept cheap and induction-transparent instead; Charge can afford aluminium because the price point supports it.

# 4. Cartridge — dual zone and magnetic retention

4.1 Zones

Two independently thermally-regulated zones per foot: heel (10×9cm) and toe (12×9cm). Each has its own induction coil pair and its own LED indicator, so heel and toe can be run at different levels — the feature Comfort's single zone deliberately doesn't offer.

4.2 Retention

Neodymium magnetic click, engineered to feel like a laptop charging connector: 3–5N to engage, 5–8N to release. Tool-free, satisfying, and rated for repeated daily removal without wearing a mechanism the way a zip or snap would. Magnets are N35 grade, rated to 80°C — comfortably above the PCM's 38–42°C operating range.

4.3 Branding window

A translucent TPU face on each cartridge, HydroWarm embossed, lets the amber PCM gel show through as it heats — the visible-technology moment the brand leans on in the heel slot. This is Charge-only; Comfort's cartridge is fully enclosed in fabric.

# 5. PCM gel fill (shared technology)

Same core chemistry as Comfort — PEG 1000, 38–42°C self-regulating phase change, food-grade, 1,000+ stable cycles. What changes on Charge is packaging (magnetic-strip TPU pouch, two zones per foot) and control (independent per-zone regulation via the four-coil plate), not the gel itself.

# 6. Induction plate (Charge variant)

| Parameter | Spec | Note |
| --- | --- | --- |
| Dimensions | 50 × 30 × 12mm | 4 coils total — 2 independent zones per foot |
| Steel disc material | Ferromagnetic only — carbon steel or 430-grade stainless | NEVER 304 stainless — non-magnetic, will not heat |
| Plate housing | ABS plastic | Non-ferromagnetic — no metal reinforcement |
| Coil frequency | 20–100kHz | Confirm exact frequency with electronics OEM |
| Surface temp (no cartridge) | < 10°C rise above ambient | Returns within 5°C of ambient within 5 minutes |
| Detection | Auto on/off per zone, ferromagnetic-disc triggered | Near-zero standby power with no cartridge present |
| Indicator | Amber LED heating / green LED at temperature | Independent per zone — 4 indicators total |

# 7. Manufacturing and sourcing

| Component | Sourcing (launch) | Note |
| --- | --- | --- |
| Aluminium channel frame | China (launch) → Poland (Year 2) | 6061 alloy, anodised |
| Slipper fabric assembly | China (launch) → Portugal (Year 2) | Minky plush + sherpa sewing |
| PCM gel fill | EU supplier preferred | PEG 1000 from Sigma Aldrich or Croda EU |
| PCM cartridge (magnetic TPU pouch) | China | Heat-welded TPU, food-grade, magnetic strip |
| Induction steel disc | China | 430-grade ferromagnetic stainless only — not 304 |
| Neodymium magnets | China | N35 grade, rated to 80°C, 3–5N engagement force |
| Induction plate PCB + coil | China — electronics OEM | Confirm coil frequency (20–100kHz) with supplier |

# 8. COGS and retail pricing

| Item | Value | Status |
| --- | --- | --- |
| COGS | Not yet quoted | Pending first factory sample — request alongside Comfort's China-track quote |
| Retail price — Single | €59 | Locked |
| Retail price — Double | €69 | Locked — same system, one wide slipper for both feet |
| Replacement cartridge (4-pack) | €19 | HW-CHARGE-CART, dual-zone with magnetic strips |

# 9. Patent claim mapping (reference only)

Charge-specific features map to the dependent claims in Patent Brief v3.0, filed against the shared Claim 1 base (wearable footwear + replaceable PCM cartridge + separate induction plate + auto-regulating temperature) that also covers Comfort. This section is for internal tracking only — claim language itself belongs in the patent brief and is attorney territory, not this factory spec.

| Claims | Feature |
| --- | --- |
| 2–3 | Access mechanism (velcro or magnetic) — Charge uses magnetic |
| 4–6 | Dual-zone cartridge + aluminium frame |
| 7–8 | Magnetic click retention detail |
| 9–10 | Multi-zone induction plate regulation |

# 10. Certification

Same standard as Comfort: IEC 60335-2-81 for the induction plate. Get a DEKRA Netherlands quote — budget €2,000–4,000 — and confirm whether Charge's four-coil plate requires separate testing from Comfort's two-coil plate before committing to tooling.

# 11. Do not change without explicit discussion

- Steel disc material: ferromagnetic only, 430-grade or carbon steel, never 304 stainless.

- PCM temperature target: 38–42°C, self-regulating via the induction plate.

- Launch sequence: Comfort first, then Charge, then Boot — Charge does not launch ahead of Comfort.

- Dual independent zones (heel/toe) — this is Charge's core differentiator from Comfort and should not be simplified to match it.

- Magnetic retention force: 3–5N engage / 5–8N release — softer risks accidental release, harder loses the tool-free feel.